|  |  | University of Sharjah College of Computing and Informatics Department of Computer Engineering |  |
| --- | --- | --- | --- |
| Senior Design Project Report Phase 1 – Senior Design Project 1 |  |  |  |
| # Title of your Project |  |  |  |
| Project Group: |  |  |  |
| U22200741 | Oumar Mamoun Ibrahim |  |  |
| U23101327 | Abdullah Jafar Abdelhamid |  |  |
| U000xxxxx | Faris Munadhil Ahmed Alagha |  |  |
| Project Examination Committee |  |  |  |
| Supervisor | Prof. or Dr. xxxxxxxxx yyyyyyyyy |  |  |
| Examiner | Prof. or Dr. xxxxxxxxx yyyyyyyyy |  |  |
| Examiner | Prof. or Dr. xxxxxxxxx yyyyyyyyy |  |  |
| Fall 20xx/20xx |  |  |  |

# Acknowledgement

3 to 4 statements. For example: We would like to thank our supervisor X for… This project cannot be accomplished without the support we get from …

# Abstract

ONLY 350-400 words in no more than two paragraphs. Describe in short, the followings:

- What is the problem in high level

- Why it is important to be addressed and it’s impact on real-life

- How you have addressed it (in high level)

- Why you address it in such a way

- How you evaluate what you propose

- What are the outcomes (performance measures/metrics) of your project

The Abstract must be bilingual English / Arabic. Not necessarily an exact translation. But, they need to convey the same information.

# Symbols, Keywords, and Abbreviations

List all abbreviations and keywords you will use in your report (MUST be alphabetically sorted )

For example:

UoS	University of Sharjah

UAE	United Arab Emirates

β	Number of devices

λ	Time interval

σ	…

…

Table of Contents

Acknowledgement	ii

Abstract	iii

Symbols, Keywords, and Abbreviations	iv

List of Figures	vii

List of Tables	viii

Problem Statement	1

1	Motivation	2

2	Introduction	3

2.1	Introduction to the topic	3

2.2	Design Criteria	4

2.3	Our Design	5

3	Technical Background	7

Triangulation and Trigonometry to calculate XY coordinates	8

4	Alternative Solutions	11

Electromagnetic Interactive Whiteboard	11

Capacitive Interactive Whiteboard	11

Resistive Interactive Whiteboard	11

Infrared Frame Interactive Whiteboard	11

Infrared Camera Based Interactive Whiteboard	11

Ultrasonic and Infrared Interactive Whiteboard	11

Comparative Evaluation-Selection of Design Approach	11

5	Proposed Design	13

5.1	Hardware Design	13

Overview of the hardware	13

High Level Block Diagram	14

Detailed Design of Hardware Components	15

5.2	Software Architecture	16

5.3	Preliminary Results	18

5.3.1	Obtained Results	18

5.3.2	Discussions	18

6	List of Constraints	19

6.1	Environmental Concerns	19

6.2	Ethical and Social Issues	19

6.3	Economic Issues	19

7	List of Specifications	20

7.1	Parts List and Estimated Budget	20

7.2	Timeline	21

References	22

Appendices	23

1	Appendix 1 – Text Style Requirements	24

1.1	Body Text	24

1.2	Chapter Headings	24

1.3	First order section headings	24

Second order section headings	24

1.4	Tables and Captions	25

1.5	Figures and Captions	25

1.6	Page numbering	26

2	Report Qualities	27

Note: The section titles in each chapter are guidelines and student may modify the titles or add other section titles and/or subtitles in consultation with his supervisor.

# List of Figures

Figure 1 – Interactive Whiteboard System Overview	1

Figure 2 – Calculating XY coordinates	1

Figure 3 – Conceptual View of the Hardware	1

Figure 4 – Hardware High Level Block Diagram	1

Figure 5 – Typical Horizontal Beam Directivity for US40KT-01	1

Figure 6 – Software Component Interaction	1

Figure 7 – Receiver Software Flowchart	1

# List of Tables

Table 1 – Comparison of Design Approach Alternatives	1

Table 2 - Parts List and Estimated Budget	1

Table 3 - Senior II Design Project Timeline	1

# Problem Statement

Write a single paragraph (maximum 2 paragraphs) that defines precisely what will be implemented in this project. This is the same problem statement that was submitted as part of your proposal; or the modified problem statement that was submitted in your progress report.

Example:

“In this project, we are going to design and Implement a system that will assist drivers to show objects in front of their cars in foggy conditions. The system will use Infrared (IR) camera and Ultrasonic Sensor with an LCD to display the Infrared Image. The Ultrasonic sensor will measure the distance to the nearest object in front of the car and display it on the LCD. The System will also include a sensor to detect fog and notify the authorities using a GSM message that will include GPS location, temperature and humidity level.”

Example 2:

“This project involves the design and implementation of a smart board system solution that replaces the traditional whiteboard to provide a more interactive environment. The main differentiating factors to consider in the design are portability, expandability, expense and power consumption. The selected design must be easily and quickly installable on any flat surface and allow use on variety of areas from small to large. The design must not be expensive to prohibit use in personal or educational settings, and allow wide range of end- users to own it. Taking portability into consideration, power consumption must be limited to provide long life-system. The target market for this design is educational and business communication purposes.”

# Motivation

Explain the motivation for your work. Answer the following questions:

- What is the purpose of your project?

- Why have you done this work: what need or problem does it address? (Briefly review relevant literature; refer to examples or other evidence that helps to show the nature, extent, or significance of the problem you define.)

- What are your goals?

The Motivation chapter must convince the reader that this report is worth reading. It must show that the report addresses an important problem and provides an exciting solution. The Motivation chapter should be between one and two pages.

Example:

Traditional whiteboards are an essential communication tool in class rooms and office environments. However, their features are limited to writing and erasing their surfaces. Nowadays, developing interactive environments with audience needs the use of visuals, animations, media and the Internet in order to approach high level of shared understanding. These features are introduced in interactive whiteboard systems which combine the simple use of boards with the power of the computer.

The smart board is an interactive whiteboard, that replaces the traditional ink pen and eraser combination with a collaborative computerized system that offers the ability to interact with information presented on the board then capture, manipulate and store data written on the board and retrieve them for future use, with the capability of integration with Internet sources or data on disk, as shown in Figure 1. It is likely to enhance the quality of education and team work by establishing an interactive space to share and discuss information.

Figure 1 – Interactive Whiteboard System Overview

# Introduction

The Introduction chapter should contain a general introduction to the topic of the project and a set of criteria that will be used to choose amongst the different design alternatives.

## Introduction to the topic

(The title of this Section changes based on your project)

Give an introduction to the topic in general. For example:

- What is meant by “smart board”

- What is “Character Recognition and what is different about Arabic Character Recognition”

- What is a “multi-touch display”

Example:

3.1 Smart Board System Overview

The smart board system is a special system which consists of a PC, projector, board and various types of sensors that are used to make the board area sensitive to touch. Sensors are either embedded within the board surface through manufacturing process or within an external structure that is used with a normal board to turn it into a smart interactive surface.

The system recognizes the user actions on the board by determining the location of the touched points. It then sends those points coordinates to the connected PC through a wired or wireless connection to be processed using special software. The user actions are then implemented on the board through the projector.

Smart board technologies vary with the types of sensors used, which determine the way of sensing the user actions and the interaction tool (special stylus or human finger) to be used by the user.

Example 2:

3.1 Arabic Character Recognition

The aim of character recognition is to transform a text document written on paper into a computer comprehensible representation, which can be easily manipulated by word processor software. It is an interface facilitating the communication between human being and machine. It also involves the problem of identifying a given form that would match with an established class after a learning process [1]. This technique is not easy because of the significant number of representations a character can have, due to the numerous fonts and styles (bold type, italic type…) that can be used.

Character recognition is of major interest because of the vastness of its application domain. Some of these applications are: archiving documents; automatic reading of checks; and automatic typing in companies.

Arabic writing, which is the topic of this study, has already been considered by some researchers. In 1980, at the University of Nancy, A. Amin was almost the first to be interested in Arabic OCR’s, and implemented the IRAC system (Interactive Recognition of Arabic Characters) [2]. Since that time, many researchers have worked on this topic in France, Syria, Algeria, Tunisia, … etc. They have used different statistical, structural, or neural approaches.

Segmentation and text recognition are two important research areas; nowadays, several text recognition devices exist on the market, but those concerning Arabic writing have a performance that needs to be improved. In this field, if there is interest in cursive writing, specific difficulties are encountered in Arabic that are not found in Latin [3]; such as:

- Arabic is a cursive type language which is written from right to left and the recognition must follow this.

- Some characters may overlap in domains covered horizontally, which are very difficult to separate (e.gأحمر يجب لحم  )

- Arabic has 28 letters, each of which can be linked in three different ways or separated depending on the case. Therefore, each character can assume 2 to 4 different forms depending on its position.

- Some characters have a very small punctuation size with respect to the rest of the text; this makes them very difficult to recognize.

- Arabic characters have different heights, therefore noise detection becomes hard.

- Words may have one or more sub-words. This is due to the fact that some characters are not connectable from the left side with the succeeding character.

## Design Criteria

In your report, you will compare between alternative designs analytically or experimentally. These discussions should show the trade-offs between the different designs. To do this, you must establish a set of criteria appropriate for your project. Then evaluate each part and/or design based on this set of criteria.

Consider various aspects:

- Cost and competitions

- Power requirements

- Implementation complexity

- Performance efficiency

Select the set of criteria that is most appropriate and practical for your project. Go back and revisit the Problem Statement. Extract the most important aspects for your project and from these extract a set of criteria

Document your criteria here.

Example:

We have established a set of criteria for the design of our smart board defined by evaluating the purposes in which the board will be used, the ability for the end-product to include more features and be commercially available in the future.

These criteria are prioritized as follows; ease of use, portability, expandability, cost, total power consumption, and parts availability. In each of the different approaches of implementing the smart board that will be discussed in this report, these criteria will be evaluated in order to choose the appropriate design.

- Ease of use - This criterion studies the ability to interact with the board and if a special stylus is needed.

- Portability - This criterion defines the ability of moving the board from one place to another or carrying it as a part of the presentation tools.

- Expandability - This criterion represents the capability of changing the size of the area of interaction with the board.

- Cost - The cost is defined in United Arab Emirates Dirhams per inch square with reference to certain available products in the market.

- Total power consumption - This is the total power consumption of the smart board system in watts.

- Parts availability - This criterion will be considered in the hardware design stage of this project. It is important when selecting the components.

## Our Design

Briefly, in one or two paragraphs, describe your planned design. Mention the parts that will be used, mention the sub-blocks, the algorithms, the tools, etc.

The purpose of this section is to introduce your design to the reader before you start discussing the technical details in the next Chapter.

In this section you would mention something like: “we will use a high-level programming language to implement our software”. Then in the next chapter you would have a section discussing C++ versus Java. If you did not include this section, the reader would be lost about why you are discussing C++/Java in the next chapter.

Each part of the design description will be elaborated in extensive detail in the next chapter.

In writing your report, keep in mind that you must always present the big picture first and then work towards the details.

Example:

In our design, we will use MATLAB 7 to implement the PSO algorithm, and test it against some classical maximization and minimization problems. Then we will use the PSO algorithm in the context of single-hop and multi-hop cellular networks. We will use the PSO to try to distribute the maximum amount of data rate to each user, with respect to the application constraints such as power budget and channel capacity.

# Technical Background

Discuss the technical background of the problem,

Thorough and concise coverage of the technical background needed to understand the problem and the solution. Intended for audiences not familiar with your topic.

Should include all the necessary information.

The Background chapter must demonstrate:

- Discussion of the mathematical, science and engineering/technical principles to be used in the project.

- Detailed discussion of each realistic constraint as they apply to the project:

  - economic factors; safety; security; reliability; aesthetics; ethics; social factors; power requirements; size and weight; IEEE, ACM, OSHA and other codes and standards.

- Discussion of the skills (programming, computational, experimental, etc.), techniques (numerical, analytical, etc), model(s) (analytical, computer, graphical, numerical) and modern tools (software, computer hardware, instrumentation, etc.) required to complete the project.

- Discussion of the design/technical methodology, including codes and standards that will be used in the project.

The contents of this chapter will vary greatly from one project to another depending on what is needed to demonstrate the above.

In this chapter you are proving to the reader that you understand the technical information needed to implement this project.

You know the technology behind each concept, and why this technology works the way it does.

Examples of possible contents for this chapter:

- If you are going to use a Microcontroller in your project, then include a section in this chapter on what a microcontroller is and how it is different compared to a microprocessor.

- If you are going to use a temperature sensor in your project, explain the theory of operation for the temperature sensor and show the equations governing its operation.

- If you are going to use an ultrasonic device to measure distance, include a section in this chapter about ultrasonic waves, the equations governing their travel, how we can calculate a distance based on ultrasonic wave travel time.

- If you are going to work on character recognition, then discuss the concepts behind character recognition, image recognition, image processing, etc.

- If you are going to use GPS in your project, explain what GPS is, how it works, the satellites, the communication, the triangulation, …

- If you are going to design an Analog-to-Digital Converter in your project, explain the concepts behind Analog-to-Digital conversion.

The content of this chapter will be guided greatly by the contents of “Our Design” section at the end of the previous chapter.

If we take the example mentioned above:

“In our design, we will use MATLAB 7 to implement the PSO algorithm, and test it against some classical maximization and minimization problems. Then we will use the PSO algorithm in the context of single-hop and multi-hop cellular networks. We will use the PSO to try to distribute the maximum amount of data rate to each user, with respect to the application constraints such as power budget and channel capacity.”

then this chapter must include at least the following sections (corresponding to the highlighted words above):

- The concept of optimization

- What is PSO

- What is MATLAB (introduction not details). What is special about version 7 (if any).

- Cellular networks, single-hop and multi-hop

- Data rate in cellular networks and what is it affected by

- Power budget for a cellular network

- Channel capacity for a cellular network.

Example:

### Triangulation and Trigonometry to calculate XY coordinates

The idea of calculating the XY coordinates in the ultrasonic and infrared smart board system is to calculate the time of arrival (TOA) of the ultrasonic signals known as triangulation. These signals which are emitted by the stylus are captured by PIC microcontroller of the receiver to calculate the distances between the stylus and each US receiver referred to as A and C in Figure 2. In order to calculate the values of XY coordinates as shown in figure 14, trigonometry formulas are used.

Figure 2 – Calculating XY coordinates

Calculating  X

- From triangle ABC, A and C are calculated using triangulation, B is fixed from hardware design.

- The angles a, b and c are found as follows:

a= cos-1(B2+C2-A22BC )       (4)

b= cos-1(A2+C2-B22AC )     (5)

c= cos-1(A2+B2-C22AB )      (6)

- Since alpha is known from the design of the receiver,

δ= 90o- α                     (7)

- Then,

θ=c- δ                           (8)

X'=Asinθ                       (9)

X=X'+Z                          (10)

Calculating  Y

- In triangle AX’Y’,

β= 90o- θ          (11)

Y'= Asinβ          (12)

Y=H-Y'                    13

Where H is the total height of the board calculated from the calibration process.

Use this chapter to demonstrate that you understand the theory behind the operation of the main components in your project.

# Alternative Solutions

What others have done to solve this problem. All related existing work should be properly described and referenced.

What are the possible different methods for solving the problem.

Here is an example of content in this chapter:

Smart Board Example

The group knew that there were many ways to implement a smart board, but they did not know which would be the most appropriate given their criteria. So, the content of this chapter for them was discussing each design method, comparing it to the criteria and coming up with a decision at the end of which design method to use.

The structure of this chapter for them included discussion of each possible design approach

### Electromagnetic Interactive Whiteboard

### Capacitive Interactive Whiteboard

### Resistive Interactive Whiteboard

### Infrared Frame Interactive Whiteboard

### Infrared Camera Based Interactive Whiteboard

### Ultrasonic and Infrared Interactive Whiteboard

### Comparative Evaluation-Selection of Design Approach

Once they had discussed and evaluated each design approach, they made their design decision on which approach to use based on the criteria they had established previously as shown in Table 1.

Table 1 – Comparison of Design Approach Alternatives

| Criteria | Interactive Whiteboard Technology |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electromagnetic | Capacitive | Resistive | IR Frame | IR Camera | IR and US |
| Easy to use | Yes | Yes | Yes | Yes | No | Yes |
| Portability | No | No | No | No | Yes | Yes |
| Expandability | No | No | No | No | Yes | Yes |
| Cost (AED/inch2) | 0.8 | 0.89 | 0.81 | 0.7 | 0.1 | 0.44 |
| Power (Watt) | 17 | 85 | 1.5 | 2.5 | Low | 0.75 |

“Based on the evaluation above, our selected design is Ultrasonic and Infrared based Smart Board System.”

The content of this chapter will be different depending on your project.

In this chapter you want to demonstrate that you have actually given consideration to what others have done and that you selected this method based on a careful evaluation of the alternatives.

# Proposed Design

Describe in detail the design you are proposing for implementing your project.

Detailed description of design

Schematics with components / Drawings / Flow diagrams

Alternative parts considered for each component in the proposed design and a design decision on which alternative will be used for each component.

Alternative algorithms considered and a decision on which algorithm will be used.

If your design involves both hardware and software, then divide this chapter into two main sections and discuss the details of hardware and software separately.

## Hardware Design

Discuss the design of the hardware.

Remember, always present the big picture first and then work towards the details.

### Overview of the hardware

Describe the hardware in general terms.

Example:

The hardware for our project consists of four main parts; transmitter, receiver, computer and the projector. The transmitter is responsible for transmitting the infrared and ultrasonic signals from the stylus in all directions. The receiver continuously reads these signals from the ultrasonic and infrared receivers to identify the position of the stylus on the board. It defines the XY coordinates, and then sends them to the computer to be processed and display user actions on the board through the projector. Figure 3 shows a conceptual view of our expected final product.

Figure 3 – Conceptual View of the Hardware

### High Level Block Diagram

Show a high level block diagram of the hardware showing conceptually how the different parts interact together. We are not talking about pin diagrams and wire-by-wire connectivity here. We are talking high level diagram.

Example:

In Ultrasonic and Infrared Interactive Whiteboards systems, there are four components involved. These are the receiver bar, transmitter pen, PC and a projector that interact together forming an interactive board out of any flat surface. Figure 10 shows a high level block diagram of the hardware system.

Figure 4 – Hardware High Level Block Diagram

### Detailed Design of Hardware Components

Discuss the design and alternatives for each component in the hardware. Conclude the section for each component with a design decision on which alternative you will be using.

Example:

Piezoelectric Ultrasonic Emitters:

These ultrasonic emitters use the piezoelectric effect to generate ultrasonic waves. Piezoelectric effect is the principle of converting energy from electrical to mechanical or vice versa by applying a pressure to a special crystal usually made of quartz [26].

In the ultrasonic emitters, the mechanical energy of the ultrasonic waves are generated by applying pressure on the crystal using electrical signals by vibrating it with a frequency of 40 KHz that is equals to the resonance frequency of the crystal.

In order to make the stylus function properly in any way it is held, the emitters are required to transmit signals in all directions. After investigating different ultrasonic emitters and by referring to [30] which proposed PT80KHz-01 with 80 KHz ultrasonic frequency, the Omni-Directional 40 kHz ultrasound transmitter US40KT-01 found to suit the design. Its horizontal directional beam angle is 360o, see Figure 5, it is small in size (1.58” x 1.58”) and has low power consumption equals to 0.15 Watts [27]. This emitter will be connected to a driver circuit to interface it with the PIC microcontroller.

The design decision: The selected ultrasonic emitter is US40KT-01.

Figure 5 – Typical Horizontal Beam Directivity for US40KT-01

## Software Architecture

Discuss the high level plan for the software.

Will the software be one major component?

Will it be made up of multiple components? If yes, then what will each component do?

Example:

Figure 6 – Software Component Interaction

Include high level flowcharts for each software component:

Figure 7 – Receiver Software Flowchart

Demonstrate that you have given good consideration and that you have developed a plan for the software portion of your project.

## Preliminary Results

Add any results (in case) you get so far from your initial experiments..

## Obtained Results

## Discussions

# List of Constraints

Document any constraints imposed by your chosen design.

Example:

- The proposed design will only work in a dark room.

- The proposed design requires a continuous connection to a power outlet. It will not function if operated from a battery.

- The proposed design will only work if the distance to the obstacle is less than 100 cm.

- If you are using a wireless signal, you should check the frequency spectrum allocation in the country and ensure no interference

## Environmental Concerns

Your report must include discussion on environmental issues related to the implementation and operation of your project.

For examples, if your project proposes equipment that uses battery, how this will affect the environment; what is the latest in recycling batteries.

## Ethical and Social Issues

Your report must include discussion on the ethical and social issues related to your project.

How people in UAE will perceive the new product. This should be discussed in the light of the social traditions

## Economic Issues

Your report must include discussions on economic issues, such as:

- How much it costs to produce the proposed device/software

- What other products in the market can compete with your proposed product

- What makes your product competitive

# List of Specifications

## Parts List and Estimated Budget

Document a completely detailed parts list.

The parts list should contain exact part numbers (and possibly sources for where to purchase the different parts).

Here is an example:

Table 2 - Parts List and Estimated Budget

| Receiver parts |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part Name | Part No. |  |  | Quantity | Supplier |  | Order Place | Price (AED) |  |
| Microcontroller | PIC16F877 |  |  | 1 | Popular electronics |  | U.A.E | 60 |  |
| US Receiver | US40KR-01 |  |  | 2 | Measurement Specialties |  | United States | 110 |  |
| IR Receiver | SFH 506-36 |  |  | 1 | Quest |  | United States | 8 |  |
| Temperature sensor | TSicTM501 |  |  | 1 | Innovative Sensor Technology |  | Switzerland | 11 |  |
| Bluetooth Module | MTS2BTSMI.R3 |  |  | 1 | MultiTech |  | United States | 240 |  |
| Total Receiver cost |  |  |  |  |  |  |  | 429 |  |
| Stylus parts |  |  |  |  |  |  |  |  |
| Part Name |  | Part No. | Quantity |  | Supplier | Order Place |  | Price (AED) |
| Microcontroller |  | PIC12C508A | 1 |  | Popular electronics | U.A.E |  | 12 |
| US Emitter |  | US40KT-01 | 1 |  | Measurement Specialties | United States |  | 55 |
| IR Emitter |  | SFH487P-36 | 3 |  | Quest | United States |  | 24 |
| Total Stylus cost |  |  |  |  |  |  |  | 91 |

| Total cost | 520 |
| --- | --- |

## Timeline

Include a timeline for implementing the project. The timeline should start right after the SDP1 presentation and should show all activities planned for SDP2. A timeline in any format is acceptable. However, you are encouraged to use project planning software such as MS Project to develop a Gantt chart that shows the time line.

Example:

Table 3 - Senior II Design Project Timeline

| Start Date | End Date | Week No. | Work to be done |
| --- | --- | --- | --- |
| 03-Jun-12 | 09-Jun-12 | 1 | Order parts |
| 10-Jun-12 | 16-Jun-12 | 2 | Driver Software: Further Research |
| 17-Jun-12 | 23-Jun-12 | 3 | Driver Software: Building |
| 24-Jun-12 | 30-Jun-12 | 4 | Driver Software: Building |
| 01-Jul-12 | 07-Jul-12 | 5 | Driver Software - Simulation and Testing |
| 08-Jul-12 | 14-Jul-12 | 6 | Transmitter Software: Building |
| 15-Jul-12 | 21-Jul-12 | 7 | Transmitter Software: Simulation and Testing |
| 22-Jul-12 | 18-Aug-12 | 8 – 11 | Ramadan |
| 19-Aug-12 | 25-Aug-12 | 12 | Eid El-Feter |
| 26-Aug-12 | 01-Sep-12 | 13 | Receiver Software: Building |
| 02-Sep-12 | 08-Sep-12 | 14 | Assembling Hardware Design |
| 09-Sep-12 | 15-Sep-12 | 15 | Assembling Hardware Design |
| 16-Sep-12 | 22-Sep-12 | 16 | Receiver Software: Building |
| 23-Sep-12 | 29-Sep-12 | 17 | Receiver Software: Simulation and Testing |
| 30-Sep-12 | 06-Oct-12 | 18 | Assembling and Integration |
| 07-Oct-12 | 13-Oct-12 | 19 | Assembling Hardware Design - Testing with Software |
| 14-Oct-12 | 20-Oct-12 | 20 | Assembling Hardware Design - Testing with Software |
| 21-Oct-12 | 27-Oct-12 | 21 | Eid Al-Adha Hiliday |
| 28-Oct-12 | 03-Nov-12 | 22 | Senior II Midterm Exam |
| 04-Nov-12 | 10-Nov-12 | 23 | Midterm Exams |
| 11-Nov-12 | 17-Nov-12 | 24 | Prototype |
| 18-Nov-12 | 24-Nov-12 | 25 | Troubleshooting |
| 25-Nov-12 | 01-Dec-12 | 26 | Report and Poster |
| 02-Dec-12 | 08-Dec-12 | 27 | National Day Holiday |
| 09-Dec-12 | 15-Dec-12 | 28 | Final Report Submission and Senior II Design Project Exam |
| 16-Dec-12 | 22-Dec-12 | 29 | Celebration the end of Senior Design |
| 23-Dec-12 | 29-Dec-12 | 30 | Fall Break |
| 30-Dec-12 | 05-Jan-13 | 31 |  |
| 06-Jan-13 | 12-Jan-13 | 32 | Final Examination |

# References

Include a list of references for your report.

If you use any material from any source, it must be referenced correctly. A reference must be included here, and it must be noted in the text of the report where you use it.

Use numbered references.

Example:

Howse, E., Hamilton, D. and Symons, L. (2000). The Effect of SMART Board Interactive Whiteboard on Concept Learning, Generation of Ideas, Group Processes and User Interaction Satisfaction. Kingston, Ontario: Queen's University.

Parmer, P. The Effects of Smart Boards on Student Learning. Retrieved from the World Wide Web on February 7th, 2012  http://www.ehow.com/list_6181886_effects-smart-boards-student-learning.html#ixzz1sPkP3FLR

Lee, J., Hong, S. and Chung, Y. (2007). Robust Tracking of Ultrasonic Pen for Handwriting Digitizer Applications. IEEE Transactions on Consumer Electronics, 54, 1098-1102.

# Appendices

Include all datasheets, extra detailed information in appendices at the end of the report.

Number the appendices sequentially and refer to them in the text of the report.

The 1st page of the appendices section should contain a list of the appendices.

Each appendix should start on a new page.

# Appendix 1 – Text Style Requirements

Follow the text styles as they are defined in this document.

## Body Text

Body text should use the “Normal” text style.

Font is 12 point Times New Roman.

Paragraphs should be Justified on both ends with 1.15 line spacing and 10 points separation between paragraphs.

## Chapter Headings

Chapter Headings should use the “Heading 1” text style.

Font is 16 point Times New Roman – Dark blue.

Justified left with 24 point separation above and 12 point separation below.

Each chapter must begin on a new page.

Chapters should be numbered sequentially. Except for the following titles which are not to be numbered:

- Table of Contents

- List of Figures

- List of Tables

- Problem Statement

- References

## First order section headings

Use “Heading 2” text style.

Font is 14 point Times New Roman – Light blue.

Justified left with 10 point separation above and 6 point separation below.

Be careful about orphan section headings. Section headings must be on the same page as the text that follows them.

First order section headings are to be numbered sequentially with the chapter number followed by a sequential section number.

### Second order section headings

Use “Heading 3” text style.

Font is 12 point Times New Roman – Italics – Light blue.

Justified left with 10 point separation above and no separation below.

Be careful about orphan section headings. Section headings must be on the same page as the text that follows them.

Second order section headings are not to be numbered.

Avoid using section headings below second order. Unless you absolutely have to.

If you must, 3rd order section headings should be 12 point Times New Roman – Black – Bold Underlined.

## Tables and Captions

Table headings should use the style “Table Caption”

From the “References” menu, select “Insert Caption”, then select “Table” from the “Label” drop-down menu. This will insert the proper caption and will make generating the “List of Tables” quite easy.

Table headings appear before the table.

Table headings use 12 Point Times New Roman font – centered.

Table headings are to be numbered sequentially throughout the report.

Be careful about orphan headings. The Table Heading must appear on the same page as the table.

Refer to the tables in you text. Use the “Cross-reference” command in the “References” menu to insert an automatically updating reference.

## Figures and Captions

Use text style “Figure” for figure, pictures, illustrations, etc.

Figure headings should use the style “Figure Caption”

From the “References” menu, select “Insert Caption”, then select “Figure” from the “Label” drop-down menu. This will insert the proper caption and will make generating the “List of Figures” quite easy.

Figure headings appear after the figure.

Figure headings use 12 Point Times New Roman font – centered.

Figure headings are to be numbered sequentially throughout the report.

Be careful about orphan headings. The Figure Heading must appear on the same page as the table.

Refer to the figures in you text. Use the “Cross-reference” command in the “References” menu to insert an automatically updating reference.

## Page numbering

Pages for the front matter (Table of Contents, List of Figures, and List of Tables) should be numbered sequentially in lower-case roman numerals (i, ii, iii, …).

Pages for the report’s text (starting with the Problem Statement) should be numbered sequentially (starting from 1) in Arabic numerals (1, 2, 3, …).

# Report Qualities

Your report will be evaluated based on the following qualities:

- Completeness

- Conciseness

- Concreteness

- Consistency

- Consideration and courtesy

- Clarity

- Usefulness

- Organization

- Style

- Format

- Grammar, Spelling

- Punctuation

Make sure to have a consistent writing style throughout the report.

- Section headings and subheadings are consistent: font, size, style, etc.

- Layout and format clear and consistent

Check the content for accuracy

All acronyms, abbreviations or terms are:

- Explained before used

- Consistent all the way through

Check your spelling and grammar

Report packaged and presented professionally